MINI-ONE

REFERENCE

Version 1 – September 2026

Edward Sicoe

github.com/esicoe/mini-one

1	CHANGELOG

| Version | Date (yyyy/mm/dd) | Change Note |
| --- | --- | --- |
| 0 | 2026/09/16 | Release |
| 1 | 2026/09/28 | Removed via shorting GND and D− |

2	SUBSYSTEM DIAGRAM

3	PCB DIAGRAM - FRONT

4	PCB DIAGRAM - REAR

5	TABLE OF COMPONENTS

| Component | MPN | Datasheet | Details |
| --- | --- | --- | --- |
| Microcontroller | RP2354B | Datasheet | Dual Cortex-M33 cores 150 MHz. Hazard3 cores are unused. |
| IMU | LSM6DSV32X | Datasheet | 6-axis, ±32 g accelerometer, ±4000 dps gyroscope. |
| Barometer | BMP585 | Datasheet | Pressure and temperature sensor. |
| Flash memory | W25Q128JVSIQ | Datasheet | 128 Mbit NOR flash. |
| microSD socket | MSD-1-A | Datasheet | - |
| GNSS | MAX-M10S-00B | Datasheet | U-blox M10 GNSS module with a passive ANT. |
| LoRa radio | RAK3172-8-SM-NI | Datasheet | 863–928 MHz LoRa with a passive ANT. |
| 3.3 V regulator | AMS1117-3.3 | Datasheet | 1 A LDO, VBATT or VBUS to 3.3V. Heatsink for VBATT power dissipation. |
| ORing diode | SS54 | Datasheet | ORing the battery onto VBUS. |
| Pyro FET | AO3400A | Datasheet | Low-side MOSFET for each of the 3 pyro channels. |
| Crystal | ABM8-272-T3 | Datasheet | MCU’s clock. |
| Core regulator inductor | AOTA-B201610S3R3-101-T | Datasheet | Inductor for onboard RP2354B 1.1V regulator |
| USB connector | USB1030-GF-P-B-B | Datasheet | USB-B female. |
| Battery connector | XT30PW-M | Product page | XT30 male battery input. |
| Pyro / arm terminals | 282834-2 | Product page | 01x02 2.54mm screw terminals. |
| Servo connectors | S3B-XH-A | Datasheet | 3-pin 2.54mm JST-XH, powered from +BATT for HV servos. |
| Antenna connectors | 132289 | Product page | SMA-F. |
| Status LEDs | KT-0805Y | Product page | Yellow LEDs. |

6	COMPONENT PROGRAMMING REFERENCES

| Component | Bus | Pins | Pico SDK reference |
| --- | --- | --- | --- |
| IMU | I²C0 | SDA - GPIO8 / PIN6 SCL - GPIO9 / PIN7 INT1 - GPIO7 / PIN4 | hardware_i2c |
| Barometer | I²C1 | SDA - GPIO10 / PIN8 SCL - GPIO11 / PIN9 | hardware_i2c |
| Flash memory | SPI0 | MISO - GPIO16 / PIN16 CS - GPIO17 / PIN17 SCK - GPIO18 / PIN18 MOSI - GPIO19 / PIN19 | hardware_spi |
| microSD | SPI1 | MISO - GPIO28 / PIN36 CS - GPIO29 / PIN37 SCK - GPIO30 / PIN38 MOSI - GPIO31 / PIN39 | hardware_spi |
| GNSS | UART0 | TX - GPIO0 / PIN77 RX - GPIO1 / PIN78 Reset - GPIO2 / PIN79 | hardware_uart |
| LoRa radio | UART1 | TX - GPIO24 / PIN25 RX - GPIO25 / PIN26 Reset - GPIO20 / PIN20 | hardware_uart |
| Status LEDs | GPIO | LED A - GPIO12 / PIN11 LED B - GPIO13 / PIN12 LED C - GPIO14 / PIN13 | hardware_gpio |
| Pyro channels | GPIO | Pyro 1 - GPIO43 / PIN54 Pyro 2 - GPIO44 / PIN55 Pyro 3 - GPIO45 / PIN56 | hardware_gpio |
| Servos | PWM | Servo 1 - GPIO32 / PIN40 (PWM8 A) Servo 2 - GPIO36 / PIN45 (PWM10 A) | hardware_pwm |
| USB | USB 1.1 | D+ - USB_DP / PIN67 D- - USB_DM / PIN66 | pico_stdio_usb |