MINI-ONE

REFERENCE

Version 1 – September 2026

Edward Sicoe

github.com/esicoe/mini-one

1	CHANGELOG

| Version | Date (yyyy/mm/dd) | Change Note |
| --- | --- | --- |
| 0 | 2026/09/16 | Release |
| 1 | 2026/09/28 | Removed via shorting GND and D− |

2	SUBSYSTEM DIAGRAM

3	PCB DIAGRAM

4	TABLE OF COMPONENTS

| Component | MPN | Datasheet | Details |
| --- | --- | --- | --- |
| Microcontroller | RP2354B | Datasheet | Dual Cortex-M33 / Hazard3 cores at 150 MHz, 520 KB SRAM, 2 MB in-package flash, 48 GPIO (QFN-80). |
| IMU | LSM6DSV32X | Datasheet | 6-axis IMU: ±32 g accelerometer, ±4000 dps gyroscope. I²C address 0x6A, INT1 on GPIO7. |
| Barometer | BMP585 | Datasheet | Pressure and temperature sensor with gel-filled port. I²C address 0x46. INT tied to GND, keep interrupts disabled. |
| Flash memory | W25Q128JVSIQ | Datasheet | 128 Mbit (16 MB) SPI NOR flash, SOIC-8. Onboard flight log. |
| microSD socket | MSD-1-A | Datasheet | Push-push microSD socket, used in SPI mode. |
| GNSS | MAX-M10S-00B | Datasheet | u-blox M10 GNSS module, passive antenna. UART at 9600 baud by default. Set dynamic model to Airborne <4 g. |
| LoRa radio | RAK3172-8-SM-NI | Datasheet | STM32WLE5-based LoRa module, 863–928 MHz, up to 22 dBm. AT commands over UART at 115200 baud. |
| 3.3 V regulator | AMS1117-3.3 | Datasheet | 1 A LDO (SOT-223) fed from VBUS. |
| ORing diode | SS54 | Datasheet | 40 V, 5 A Schottky. ORs the battery onto VBUS. |
| Pyro FET | AO3400A | Datasheet | 30 V N-channel MOSFET (SOT-23). Low-side switch for each pyro channel (×3). |
| Crystal | ABM8-272-T3 | Datasheet | 12 MHz, 10 pF load. System and USB clock. |
| Core regulator inductor | AOTA-B201610S3R3-101-T | Datasheet | 3.3 µH (0806) for the RP2354B internal 1.1 V regulator. |
| USB connector | USB1030-GF-P-B-B | Datasheet | USB-B receptacle, through-hole. 5 V input and USB 1.1. |
| Battery connector | XT30PW-M | Product page | XT30 male, horizontal PCB mount. 2S LiFe input. |
| Pyro / arm terminals | 282834-2 | Product page | 2-position, 2.54 mm terminal block (×4). |
| Servo connectors | S3B-XH-A | Datasheet | 3-pin JST-XH, side entry (×2). Powered from +BATT. |
| Antenna connectors | 132289 | Product page | SMA edge-mount jack (×2) for GNSS and LoRa. |
| Status LEDs | KT-0805Y | Product page | 0805 yellow LED (×3). |

5	COMPONENT PROGRAMMING REFERENCES

| Component | Bus | Pico SDK reference |
| --- | --- | --- |
| IMU | I²C0 | hardware_i2c |
| Barometer | I²C1 | hardware_i2c |
| Flash memory | SPI0 | hardware_spi |
| microSD | SPI1 | hardware_spi |
| GNSS | UART0 | hardware_uart |
| LoRa radio | UART1 | hardware_uart |
| Status LEDs | GPIO | hardware_gpio |
| Pyro channels | GPIO | hardware_gpio |
| Servos | PWM | hardware_pwm |
| USB | USB | pico_stdio_usb |